GB/2799E/B(U) (Rev.8)

CERTIFICATE OF APPROVAL OF PACKAGE DESIGN 
FOR THE CARRIAGE OF RADIOACTIVE MATERIAL

This is to certify that for the purposes of the Regulations of the International Atomic Energy Agency

The Competent Authority of Great Britain in respect of inland surface transport, being the Office for Nuclear Regulation;

The Competent Authority of the United Kingdom of Great Britain and Northern Ireland in respect of sea transport, being the Secretary of State for Transport;

The Competent Authority of the United Kingdom of Great Britain and Northern Ireland in respect of air transport, being the Civil Aviation Authority; and

The Competent Authority of Northern Ireland in respect of road transport, being the Department of Agriculture, Environment and Rural Affairs - Northern Ireland

approve the package design specified in Section 1 of this certificate, as submitted for approval by Croft Associates Ltd (see Section 5)

as: Type B(U)

by: air, road, rail, sea and inland waterway.

Packaging identification: SAFKEG Package design No 2799E

Packages manufactured to this design meet the requirements of the regulations and codes on pages 3 and 4, relevant to the mode of transport, subject to the following general condition and to the conditions in the succeeding pages of this certificate.

In the event of any alteration in the composition of the package, the package design, the management system(s) associated with the package or in any of the facts stated in the application for approval, this certificate will cease to have effect unless the Competent Authority is notified of the alteration and the Competent Authority confirms the certificate notwithstanding the alteration.

Expiry Date: This certificate cancels all previous revisions and is valid until the end of May 2028 (see Section 5).

COMPETENT AUTHORITY IDENTIFICATION MARK: GB/2799E/B(U)

Signature:	Date of Issue: 29 June 2023

Geoff Frackelton, Head of Transport Competent Authority

Office for Nuclear Regulation

Redgrave Court, Merton Road

Bootle, Merseyside

L20 7HS

on behalf of the Office for Nuclear Regulation; the Secretary of State for Transport; the Civil Aviation Authority; and the Department of Agriculture, Environment and Rural Affairs - Northern Ireland.

This certificate does not relieve the consignor from compliance with any requirement of the government of any country through or into which the package will be transported.

REGULATIONS GOVERNING THE TRANSPORT OF RADIOACTIVE MATERIALS

INTERNATIONAL

International Atomic Energy Agency (IAEA)

SSR-6 Regulations for the Safe Transport of Radioactive Material 2018 Edition

United Nations Economic Commission for Europe (UNECE)

Agreement concerning the International Carriage of Dangerous Goods by Road (ADR) 2021 Edition (until end June 2023) or Agreement concerning the International Carriage of Dangerous Goods by Road (ADR) 2023 Edition

Intergovernmental Organisation for International Carriage by Rail (OTIF)

Regulations concerning the International Carriage of Dangerous Goods by Rail (RID) 2021 Edition (until end June 2023) or Regulations concerning the International Carriage of Dangerous Goods by Rail (RID) 2023 Edition

International Maritime Organization (IMO)

International Maritime Dangerous Goods (IMDG) Code 2020 Edition incorporating Amendment 40-20

International Civil Aviation Organization (ICAO)

Technical Instructions for the Safe Transport of Dangerous Goods by Air 2023-2024 Edition

UNITED KINGDOM

ROAD

GREAT BRITAIN ONLY:

The Energy Act 2013 (2013 c. 32); The Carriage of Dangerous Goods and Use of Transportable Pressure Equipment Regulations 2009 (SI 2009 No. 1348); The Energy Act 2013 (Office for Nuclear Regulation) (Consequential Amendments, Transitional Provisions and Savings) Order 2014 (SI 2014 No. 469)

NORTHERN IRELAND ONLY:

The Carriage of Dangerous Goods and Use of Transportable Pressure Equipment Regulations (Northern Ireland) 2010, (SR 2010 No 160)

RAIL

GREAT BRITAIN ONLY:

The Energy Act 2013 (2013 c. 32); The Carriage of Dangerous Goods and Use of Transportable Pressure Equipment Regulations 2009 (SI 2009 No. 1348); The Energy Act 2013 (Office for Nuclear Regulation) (Consequential Amendments, Transitional Provisions and Savings) Order 2014 (SI 2014 No. 469)

SEA

British registered ships and all other ships whilst in United Kingdom territorial waters:

The Merchant Shipping Act 1995 (1995 c. 21); The Merchant Shipping (Dangerous Goods and Marine Pollutants) Regulations 1997 (SI 1997 No. 2367); Merchant Shipping Notice MSN 1906 (M) The carriage of dangerous goods and marine pollutants:  Amendments to international standards

AIR

The Air Navigation Order 2016 (SI 2016 No. 765); The Air Navigation (Dangerous Goods) Regulations 2002 (SI 2002 No.2786)

DESIGN SPECIFICATION

Package Design

The package design specification shall be in accordance with Design Safety Report for SAFKEG Design No 2799E, DSR 2799E/B(U)-96 Issue M, dated 20 July 2022, and modifications to the package design approved by the authorities named on page 1 of this certificate under the established modifications procedure.

Design Drawings

The design is specified in the following drawings.

| Design No. | Title (number of components) | Drawing / Drawing List | Issue |
| --- | --- | --- | --- |
| 2799E | DRAWING LIST FOR DESIGN No 2799E PACKAGINGS – ALL BUILDS | DL-1C-5407 | F |
| And Innermost Packaging Arrangement 1 |  |  |  |
| -- | Bayer Contents (2799E) Licensing Drawing | 0C-7598 | A |
| Or Innermost Packaging Arrangement 2 |  |  |  |
| -- | Bayer Contents (2799E) Licensing Drawing | 0C-7863 | A |
| -- | Wheaton 3.0ml Glass Vial | W986277NG | C |
| Or Innermost Packaging Arrangement 3 |  |  |  |
| -- | Bayer Contents 2799E (Zr Vial option) Licensing Drawing | 0C-8099 | A |
| -- | Zirconium Vial | N3D020399A356 | B |

Package Description and Materials of Manufacture

The packaging consists of an outer double skin insulated keg, an insulating cork liner, a re-sealable containment can, an inner lead shielding pot and aluminium spacers. The outer keg is made of stainless steel filled with insulating phenolic resin foam and with a flat stainless-steel lid attached by eight stud/nut arrangements. The inner containment can and lid are manufactured from stainless steel, closed by a threaded retaining ring and sealed with a double EPDM O-ring. See Appendix 1 for package illustration.

There are three acceptable designs of innermost packaging arrangements. The contents are held centrally within the containment vessel by aluminium spacers and are shielded by a lead pot. The innermost packaging parts identified in arrangement 1 are not interchangeable with those of packing arrangements 2 and 3. The lead pot, aluminium spacers and aluminium can are interchangeable between Innermost Packing Arrangement 2 and 3.

Package Dimension and Weights

Nominal dimensions: 425mm diameter x 540 mm high

Maximum authorised gross weight: 67.5 kg

Authorised Contents

The authorised radioactive contents for Innermost Packing Arrangement 1 are:

        - 20 GBq of Actinium 227 as a solid salt,

        - daughter products and

        - trace amounts (< 1%) of other radionuclides due to the production method.

The authorised radioactive contents for Innermost Packing Arrangement 2 and 3 are:

        - 20 GBq of Actinium 227 and 1 GBq of Actinium 225 as a solid salt,

        - daughter products and

        - trace amounts (< 1%) of other radionuclides due to the production method.

Restriction on Contents

The package is designed to carry a maximum heat load of 10W.

Containment System

The containment system consists of:

        - Containment Vessel (CV) design No 2812 - The containment Vessel Design No. 2812, the CV lid and the inner O-ring seal provides absolute containment for the radioactive contents. A second (outer) O-ring is used to facilitate leakage testing. The CV lid is retained by a single screw threaded ring (M143.5 x 4.0).

        - Keg Design No. 2799 - The keg (Design No. 2799) holds and provides impact and thermal protection of the 2812 CV (Design No. 2812). The keg is fitted with a flat lid that is secured by eight M12 studs and nuts. These bolted closures form the positive fastening device of the keg closure.

use of package

Information Provided in Safety Report on Use of Packaging

The packaging shall be used and handled in accordance with Croft Associates Ltd. Packing and Handling Instructions CPI 013 Issue V.

The packaging shall be maintained in accordance with Croft Associated Ltd. Serviceability Procedure CSP 006 Issue K.

Actions Prior to Shipment

Administrative controls shall ensure that the contents are in accordance with Section 1 of this certificate, and that the consignor and consignee hold a copy of the certificate and instructions on the use of the packaging.

Supplementary Operational Controls

None required.

Emergency Arrangements

Before shipment takes place, adequate emergency arrangements must be made, copies of which shall be supplied to the GB Competent Authority on demand.

Within Great Britain, if the consignor’s own, or other approved emergency plans, cannot be initiated for any reason, then the police shall be informed immediately.

management systems

The management system(s) assessed as adequate in relation to this design by the authorities named on page 1 of this certificate, at the date of issue, are as specified in Design Safety Report for SAFKEG Design No 2799E, DSR 2799E/B(U)-96 Issue M, dated 20 July 2022, referred to in Section 1 above, and comprise the following:

Croft Associates Ltd Quality Management System.

No alteration may be made to any management system confirmed as adequate in relation to this design, unless:

the authorities named on page 1 of this certificate have confirmed the amended management system is adequate prior to implementation or use; or

the alteration falls within the agreed change control procedures set out in the management system(s).

Other management systems for design, testing, manufacture, documentation, use, maintenance, inspection, transport and in-transit storage operations may be used providing they comply with international, national or other standards for management systems agreed as acceptable by the authorities named on page 1 of this certificate.

ADMINISTRATIVE INFORMATION

Related Approvals

Not applicable.

Shipment Approval

Not applicable.

Packaging Serial Numbers

For the purpose of compliance with ADR / RID, the owner of the packaging shall be responsible for informing ONR of the serial number of each packaging manufactured to this design.

CERTIFICATE STATUS

Design approval issued to:

Croft Associates Ltd

F4 Culham Science Centre

Abingdon

Oxfordshire

OX14 3DB

UK

| Issue / Revision Number | Date of Issue | Date of Expiry | Reason for Revision |
| --- | --- | --- | --- |
| 4 | 18 June 2001 | 31 March 2004 | Renewal as GB/2799E/B(U)F-85. |
| 1 | 24 June 2004 | 30 June 2007 | Design approval issued under new regulations. Fissile content removed and minor amendments/clarification. |
| 2 | 5 March 2008 | 30 June 2010 | Renewal with minor changes (1 year extension issued on 21 June 2010). |
| 3 | 30 June 2011 | 30 June 2016 | Renewal. |
| 4 | 29 June 2016 | 31 Dec 2016 | Short Term Certificate with limitations on contents. |
| 5 | 11 July 2017 | 31 July 2022 | Renewal for Ac-227 only. |
| 6 | 6 September 2018 | 31 July 2022 | Inclusion of Croft Modification CN048 |
| 7 | 10 February 2020 | 31 July 2022 | Inclusion of Croft Modification CN066 |
| 8 | 29 June 2023 | 31 May 2028 | Renewal |

Appendix 1

Figure 1 - Keg and containment vessel

Figure 2 - Innermost Packaging Arrangement 1

Figure 3 - Innermost Packaging Arrangement 2

Figure 4 - Innermost Packaging Arrangement 3